The Dallas Semiconductor Information Button, or iB, is a	the port line tied to Vcc (+SV) via a (nominal) 4K7 resistor.

non-volatile RAM, with a serial interface, carried in a two-	The iB sends data by simply generating current pull when it

contact housing, often using a button-battery style enclosure.	wants to send a zero, and the current pulls the resistor vult

The name of these devices has itself undergone a sort of evo-	age down far enough to be sensed as a low on the (input)

lution at Dallas; originally they were called "Touch Memo-	port line. To send a "one" the iB, of course, does not need to

ries," then they were known as "Information Buttons," and	do anything.

then as "Autoldentification" devices. Of all this, we at jarrah		The serial protocol involves commands being sent out to

Computers have stuck with the "Information Button" no-	the ill, and the iB sends the required data back over the same

menclature. There is an entire family of devices in the iB range,	wire, so the processor must switch the port line to input mode

all denominated DS19XX. The family include memory of	before the iB starts sending the data. As reading data (and thus

various sizes, one contains a timer, and one a thermometer.	switching the direction of the port line) is involved in all in

Jarrah Computers have primarily used the DS1996 (64Kbit	teractions between a processor and the iB-and as we must

NV RAM), and this article is based on code which interfaced	ensure that we are never driving the line when the iB is-syn

to the 1996, but we believe the processes are similar for all	chronous dynamic control of the bi-directional port line's di

members of the family.	rection is a critical part of the interface to the Dallas iBs.

Dallas have promoted the iBs as "attaching digital data to		The most important aspect of the serial protocol coding is

physical objects"-their intended applications being in in-	timing. The processor must drive and sense the line during

dustry to track, say, gas cylinders, shipping containers, and	the allocated time slots, so the time-critical routines must be

the like. To this end, there is quite a bit of information in the	worked out almost on a cycle-by-cycle basis.

Dallas data sheets (available on-line at www.ibutton.com) on

how to run connections to the iB wherever it is located on its	1. M Interface Details

"physical object"-sometimes a couple of metres of wire are		There are four basic processes in dealing with the iBs:

1 used; some hookups use metal straps, or a set of cables, and

connectors to wire the iB from wherever it is physically lo-	1. 1 Reading "presence'~--refer to Figure One

cated back to a reading/writing device.		To read ill presence, the port line (in output mode) must

This is worth noting, as any interface code must almost	be driven low for greater than 480 [tS, and then the port line

presume that the contacts to the ill may be momentarily high	must be sampled (in input mode), starting between 15 and

resistance-that contacts may be making and breaking as we	60 RS after the low pulse is released, and the ill will drive the

try to communicate. The retries in the code are primarily to	line for between 60 and 240 [tS. If an iB is present, there will

handle momentary breaks in connection. The example given	be a "low" generated by the iB; if no iB is present, the line

at the end of this article, on backup memories, is an example	will of course stay high due to the tie-high resistor.

of attaching digital data (in iB) to a CPU board, which is, in

the end, a physical object!	1.2. Reading a data bit-refer to Figure Two

The memory in the iB is organised into a set of 32-byte		To read an ill data bit, the port line (in output mode) must

pages, plus a 32-byte scratchpad, via which all reads and writes	be driven low for between 1 and 15 gS (basically, as short as

are mediated. Furthermore, Dallas recommend that the data	possible!), then the port line must be sampled (in input mode),

be formatted into what they call Universal Data Packets, which	starting as soon as the low pulse is released, and the ill will

is a count byte, the data, then a two-byte CRC, meaning that	drive the line for a window of 15 [tS. If the ill data is zero,

only 29 bytes of actual data can be stored in each 32-byte	there will be a "low" generated by the iB; if the ill data is one,

page. The UDP, including checksum, provides an extra check	the line is left high.

that good data has been read and is, in our view, well worth

the extra effort.	1.3. Writing a "onebit-refer to Figure Three

There are two modes for dealing with ills, called regular		To write an ill data bit "one," the port line (in output mode)

and overdrive speeds by Dallas-we will only be dealing with	must be driven low for between 1 and 15 ~tS (basically, as

the regular speed, as overdrive requires processors which are	short as possible!), then the port line is left high for the re

faster than a (2 MHz) 68HCl 1-they typically deal in 2-10RS	mainder of the 60 [tS window.

periods, which is faster than some 68HCI1 instructions!

The bi-directional serial interface is implemented on a	1.4. Writing a "zero" bit-refer to Figure Four

single active wire, and ground. The recommended processor		To write an iB data bit "zero," the port line (in output

interface is a single bi-directional port line (and ground), with	mode) must be driven low for the entire 60 RS window.

48	Forth Dimensions XXI.1,2